Hyper-V Network and OPNsense Interface Reconfiguration

IT Standard Operating Procedure

System: CS-HyperV1

Classification: Internal SOC Infrastructure

Effective Date: 12/05/2025

Version 1.0

Date: 12/05/2025

Department Responsible: Infrastructure / Security Engineering

| Version History |  |  |  |
| --- | --- | --- | --- |
| Version No. | 1.0 | Current Version Date | 12/05/2025 |
| Effective Date | 12/05/2025 | Expiration Date | N/A |
| Responsible Person | Infrastructure Administrator | Signature | Pending |
| Writer | Joshua Sartain | Approval | Pending |

Review Procedure

Review this SOP after any Hyper-V virtual switch modification, OPNsense interface reassignment, or physical NIC recabling event. Perform a formal review at least every 6 months. The Infrastructure Administrator is responsible for validating technical accuracy and updating this document.

Purpose

This SOP documents the corrective reconfiguration of the Hyper-V host CS-HyperV1 and the associated OPNsense firewall to establish proper separation between WAN, LAN, and internal SOC network segments. The change removes an improper dual-purpose NIC design, restores a valid WAN path for OPNsense, and defines the remaining action required to extend the internal SOC LAN to classroom-connected physical hosts.

Scope

Applies to Hyper-V virtual switch configuration, physical NIC bindings, OPNsense interface assignments, routing and NAT boundaries, and connectivity between internal SOC virtual machines and 22 physical classroom systems connected to the lab switch.

Roles And Responsibilities

| Role | Responsibility |
| --- | --- |
| Infrastructure Administrator | Performs Hyper-V switch changes, physical NIC binding updates, and host-level validation. |
| Firewall Administrator | Assigns and validates OPNsense interfaces, gateway behavior, NAT, and DHCP settings. |
| Systems Administrator | Validates VM connectivity, internal service reachability, and management access. |
| Lab Support Staff | Confirms classroom host connectivity after LAN extension is completed. |
| Change Approver | Reviews change scope, rollback readiness, and post-change validation results. |

Keywords And Definitions

| Keyword | Definition |
| --- | --- |
| Hyper-V vSwitch | Virtual switch used by Hyper-V to connect VMs, host adapters, and physical NICs. |
| External vSwitch | Hyper-V switch type bound to a physical NIC and able to pass traffic externally. |
| Internal vSwitch | Hyper-V switch type allowing communication between the host and VMs only. |
| OPNsense | Virtual firewall/router platform providing routing, NAT, DHCP, and policy enforcement. |
| Layer 2 Domain | Shared switching domain in which endpoints can communicate directly without routing. |

Essential Business Functions

This configuration supports internet access for internal SOC virtual machines, upstream routing through OPNsense, predictable NAT behavior, and controlled access from classroom systems to SOC-hosted services. Improper NIC sharing or incorrect switch binding can cause WAN loss, overlapping traffic domains, and isolation of physical student systems from internal resources.

Procedures

Completed corrective actions: 1. Reconfigure SOC-USER from Internal to External and bind it to the Broadcom NetXtreme E-Series Dual-Port 25Gb SFP28 adapter. 2. Assign OPNsense WAN to hn0, enable the interface, and obtain upstream DHCP addressing. 3. Remove Embedded NIC2 from the prior mixed WAN/LAN role and clear conflicting bindings. 4. Validate stable WAN, gateway, DNS, and outbound NAT behavior for internal VMs. 5. Pending: convert SOC-LAN from Internal to External and bind it to Embedded NIC2 so classroom systems and OPNsense LAN share one Layer 2 segment.

| Procedure Name | Task | Person Responsible |
| --- | --- | --- |
| Reconfigure WAN vSwitch | Change SOC-USER from Internal to External and bind it to the SFP28 NIC. | Infrastructure Administrator |
| Reassign OPNsense WAN | Assign WAN to hn0, enable DHCP, and validate gateway and DNS receipt. | Firewall Administrator |
| Validate Interface Roles | Confirm WAN, SOCMGMT/LAN, SOCUSER, SOCDMZ, and unused interface mapping. | Firewall Administrator |
| Remove Dual-Use NIC Design | Remove NIC2 from the prior mixed WAN/LAN role and clear conflicting bindings. | Infrastructure Administrator |
| Validate Connectivity | Test WAN reachability, NAT behavior, internal LAN communication, and VM internet access. | Systems Administrator |
| Complete LAN Extension | Bind SOC-LAN to Embedded NIC2 so classroom systems and the firewall share one LAN segment. | Infrastructure Administrator |

A flowchart can help to show what actions to take to complete a helpdesk ticket based on certain conditions.

Troubleshooting Procedures

If OPNsense WAN does not receive DHCP, verify that SOC-USER is an External vSwitch bound to the Broadcom NetXtreme E-Series SFP28 NIC and confirm the upstream network is providing DHCP. If internal VMs lose internet access, verify WAN gateway presence, outbound NAT state, LAN-to-WAN firewall rules, and VM attachment to the intended vSwitch. If student systems cannot reach SOC resources, confirm SOC-LAN is externally bound to Embedded NIC2, the classroom switch is physically connected to that NIC, and no conflicting DHCP server exists on the same subnet.

Checklists

Add all checklists used for processes and describe filing systems for completed checklists.

Pre-Change: record current Hyper-V vSwitch bindings, OPNsense interface assignments, and confirm console access to both the host and firewall. Post-Change: verify WAN on hn0, DHCP lease receipt, default gateway visibility, DNS resolution, VM internet access, and LAN convergence after SOC-LAN is externally bound.

Service Continuity

Are backup and restore tasks defined to ensure service availability and continuity during and after incidents?

Maintain direct console access to CS-HyperV1 and the OPNsense VM throughout implementation. Preserve prior NIC and vSwitch mapping notes so the environment can be returned to the last known working state if routed connectivity fails.

Escalations

Low: documentation or labeling discrepancy -> Infrastructure Administrator Medium: classroom systems unable to reach SOC services -> Systems Administrator / Infrastructure Administrator High: WAN outage or loss of routed connectivity -> Firewall Administrator / Change Approver Critical: host isolation or loss of firewall access during production use -> Infrastructure Lead / immediate rollback

Record Management

Retain Hyper-V switch screenshots or exports, OPNsense interface assignment screenshots, IP addressing notes, validation ping results, and change approval records in the internal infrastructure repository with restricted access.

Maintenance

After patching or hardware changes, verify NIC bindings, OPNsense interface mapping, vSwitch roles, and DHCP/NAT behavior. Review the logical diagram whenever interface assignments change.

Equipment, Hardware, And Software

Hardware: CS-HyperV1, Broadcom NetXtreme Gigabit Ethernet embedded NICs, Broadcom NetXtreme E-Series Dual-Port 25Gb SFP28 adapter, and the classroom switch serving 22 student hosts. Software: Microsoft Hyper-V, OPNsense firewall, internal SOC virtual machines, and classroom-connected physical endpoints.

Configuration

Original State: Embedded NIC2 was used simultaneously for WAN uplink and internal LAN traffic, SOC-USER was Internal Only, OPNsense had no clean WAN path, and classroom hosts were isolated from the intended internal LAN despite sharing the same subnet. Current State: SOC-USER is now bound externally to the SFP28 NIC, OPNsense WAN is hn0 with DHCP, WAN/NAT behavior is stable, and NIC2 is no longer serving a mixed transport role. Pending State: SOC-LAN must be converted from Internal to External and bound to Embedded NIC2 so OPNsense LAN (192.168.1.1), internal SOC VMs, and classroom hosts share one Layer 2 domain.

Diagrams

Current Post-Change Logical Diagram Upstream Network ¦ [ SFP28 NIC ] ¦ Hyper-V vSwitch: SOC-USER ¦ OPNsense WAN (hn0) DHCP 192.168.107.x / NAT ----------------------------------------------------- Hyper-V Internal vSwitch: SOC-LAN (Internal) ¦ OPNsense LAN (hn1) 192.168.1.1 ¦ Internal SOC VMs ----------------------------------------------------- Physical Classroom Switch (22 Hosts) ¦ Embedded NIC2 [Not currently mapped to SOC-LAN] Intended Final State Upstream Network ¦ [ SFP28 NIC ] ¦ Hyper-V vSwitch: SOC-USER ¦ OPNsense WAN (hn0) DHCP 192.168.107.x / NAT ----------------------------------------------------- Hyper-V vSwitch: SOC-LAN (External) ¦ Embedded NIC2 ¦ Physical Classroom Switch ¦ ¦ Student Hosts Internal SOC VMs \ / \ / OPNsense LAN (hn1) 192.168.1.1

Service Review

Issues identified during review: mixed WAN/LAN duties on one physical adapter, classroom systems isolated from the intended SOC LAN, and overlapping use of the same IP subnet across separate switching domains. Status: WAN corrected, NAT stabilized, and LAN extension to physical classroom hosts remains pending.

Service Catalogue

Supports SOC lab connectivity, internal domain and management services, classroom-connected lab access, firewall routing and segmentation, and VM internet access through controlled NAT.

Change Process

1. Record current-state Hyper-V and OPNsense configuration. 2. Obtain change approval. 3. Implement vSwitch modifications in Hyper-V. 4. Reassign and validate interfaces in OPNsense. 5. Perform post-change testing and record results. 6. Update diagrams and the SOP. 7. Schedule or perform the final SOC-LAN to Embedded NIC2 binding.

References

Microsoft Hyper-V Virtual Switch Manager OPNsense interface assignment and gateway configuration Internal infrastructure addressing plan Host NIC inventory and adapter mapping

Signatures

Obtain signatures from employees to confirm that they have read and understood procedures.

| Staff Member Name | Signature | Date |
| --- | --- | --- |
| Infrastructure Administrator |  |  |
| Firewall Administrator |  |  |
| Change Approver |  |  |