# Standard Operating Procedure (SOP)

Hyper-V Host Installation and Initial Setup

Applies to Hyper-V hosts prepared for the SOC lab, CMMC-aligned training, and Zero Trust validation environments.

| SOP ID | ENG-HV-001 | Version | 2.0 |
| --- | --- | --- | --- |
| Document Status | Draft for controlled use | Effective Date | 10/06/2025 |
| Prepared By | Joshua Sartain | Original Source | Nicholas Jones / prior lab draft |
| Department | CVTech Cyber Engineering | Location | CVTech El Reno - Room 1063 |

Document intent. This SOP supersedes the earlier mixed endpoint/VM draft and now focuses on the actual task: preparing a Windows Hyper-V host, enabling the Hyper-V role, configuring storage and networking baselines, and validating the host so downstream lab VMs can be deployed consistently.

# 1. Purpose

This procedure defines the standard method for preparing a Windows host for Hyper-V use in the SOC lab environment. It provides a repeatable process to verify platform readiness, install the Hyper-V role, configure host-level storage and virtual networking, and validate the host before virtual machines are deployed.

The objective is to ensure the Hyper-V host is secure, stable, and aligned to lab design requirements before endpoint, server, or security workloads are introduced.

# 2. Scope

This SOP applies to physical or primary lab hosts intended to run Hyper-V for the SOC.lab environment. It covers host preparation, BIOS and operating system readiness checks, Hyper-V role installation, initial storage configuration, virtual switch planning, and post-install validation.

This SOP does not cover guest operating system installation, domain join of virtual machines, Wazuh agent deployment inside guests, or endpoint lifecycle management. Those activities belong in separate SOPs.

# 3. Roles and Responsibilities

System Administrator

Verify hardware readiness, BIOS virtualization settings, Windows edition, storage availability, and patch level

Install and validate the Hyper-V role

Configure host storage paths and initial virtual switch design

SOC Engineer / Instructor

Provide approved host naming, network design, and VM placement standards

Validate the host meets lab and security requirements before VM deployment

Approve deviations from the standard build

Authorized Technician / Student Operator

Follow this SOP exactly when assigned to build or refresh a Hyper-V host

Escalate errors, unsupported hardware findings, or policy conflicts to the instructor or administrator

Lab Management

Maintain approved source media, license status, and hardware inventory

Ensure build activities align with safety, security, and training objectives

# 4. Prerequisites

Permissions

Local Administrator rights on the Windows host

Access to approved installation media and lab documentation

Hardware

64-bit processor with Second Level Address Translation (SLAT)

Hardware virtualization enabled in BIOS/UEFI (Intel VT-x/VT-d or AMD-V/RVI as applicable)

Minimum 4 logical CPU cores recommended

Minimum 8 GB RAM recommended; 16 GB or more preferred for multi-VM use

Minimum 100 GB free SSD-backed storage recommended for lab workloads

Software

Supported edition of Windows 10/11 Pro, Enterprise, or Windows Server with Hyper-V support

PowerShell and Hyper-V management tools available

Approved ISO images or gold images for downstream guest creation

Network

Connection to SOC-LAN or approved isolated lab segment

Required upstream access for updates, management, and repository reachability where permitted

# 5. Required Components

Windows host platform supporting Hyper-V

Hyper-V role and Hyper-V Management Tools

Local or secondary storage path for VHDX, checkpoints, and configuration files

Planned virtual switch design (External, Internal, and/or Private as required)

Administrative tools: PowerShell, Hyper-V Manager, Event Viewer, optional Failover Cluster tools if the host is part of a broader design

# 6. Procedure

Follow the phases below in order. Do not begin guest VM creation until all host validation checks in Phase 7 pass.

## 6.1 Environment Preparation

Confirm the host is the correct asset and record hostname, serial number, physical location, and intended role.

Log on with a local or domain account that has local administrative rights.

Confirm the host is connected to the approved lab network and power source, and that storage health is normal.

Open System Information and verify Windows edition, build level, installed memory, and processor details.

Apply current Windows updates if the host is behind the approved patch baseline.

## 6.2 Verify BIOS/UEFI Virtualization Readiness

Reboot to BIOS/UEFI.

Enable hardware virtualization features, including Intel Virtualization Technology / VT-d or AMD-V / IOMMU equivalents.

Ensure Data Execution Prevention / NX / XD is enabled.

Save changes and return to Windows.

From an elevated PowerShell session, run systeminfo and verify Hyper-V requirements show as available.

## 6.3 Validate Storage and Host Layout

Confirm there is sufficient local or attached storage for the planned VM footprint.

Create or verify standardized directories for VM configuration files, virtual hard disks, and ISO storage. Recommended example:

C:\Hyper-V\VMs

C:\Hyper-V\VHDX

C:\Hyper-V\ISO

If a secondary disk is used, align the storage path with lab standards and document the drive letter or mounted volume path.

Ensure NTFS permissions remain restricted to administrators and Hyper-V service requirements.

## 6.4 Install the Hyper-V Role

Open an elevated PowerShell session.

Install the role and management tools using the approved method:

Method A: GUI - Control Panel > Turn Windows features on or off > select Hyper-V, Hyper-V Management Tools, and Hyper-V Platform.

Method B: PowerShell - Enable-WindowsOptionalFeature -Online -FeatureName Microsoft-Hyper-V -All

Restart the host when prompted.

After reboot, open Hyper-V Manager and confirm the local host loads without errors.

## 6.5 Configure Default Hyper-V Paths

In Hyper-V Manager, open Hyper-V Settings for the local host.

Set Virtual Hard Disks to the approved VHDX path.

Set Virtual Machines to the approved VM configuration path.

If enhanced session mode is permitted for the lab, configure it per policy; otherwise leave defaults unchanged.

Document the configured paths in the build record.

## 6.6 Configure Initial Virtual Switches

Open Virtual Switch Manager.

Create the required switch types based on the lab design:

External switch for upstream or SOC-LAN connectivity

Internal switch for host-to-VM communication where required

Private switch only if isolated VM-only communication is explicitly needed

Bind the external switch to the correct physical NIC. Do not attach multiple production-equivalent roles to the same adapter without an approved design.

If the management OS must share the adapter, explicitly enable it; otherwise leave the host isolated from that switch by design.

Label switches clearly using operational names, for example SOC-LAN, SOC-WAN, MGMT, or LAB-INT.

## 6.7 Apply Initial Host Hardening

Rename the host to the approved naming convention if not already completed.

Join the host to the correct domain only if the architecture requires a domain-joined Hyper-V host.

Enable and verify Windows Defender Firewall profiles unless an approved security control replaces them.

Disable unnecessary software and remove unapproved consumer applications.

Install required host monitoring or logging tools only if approved for host-level monitoring.

## 6.8 Validation and Functional Testing

Open Hyper-V Manager and verify the host status is healthy.

Create a temporary test VM or use a validation VM template to confirm the host can create, store, and start a guest.

Confirm the selected virtual switch provides the expected connectivity.

Verify Event Viewer does not show critical Hyper-V or storage initialization errors.

Run basic connectivity testing to the domain, management network, and approved upstream services as applicable.

Remove the temporary validation VM if it is not part of the final build.

## 6.9 Handover to VM Deployment Phase

Record the host as build-complete only after validation succeeds.

Capture screenshots or exports of the Hyper-V settings, storage paths, and virtual switches.

Notify the SOC engineer or instructor that the host is ready for guest deployment activities.

Proceed to the separate VM deployment SOP for Windows 10/11 lab endpoints or server roles.

# 7. Validation Checklist

| Check | Expected Result | Status |
| --- | --- | --- |
| Windows edition supports Hyper-V | Edition is Pro, Enterprise, or Server and feature installs successfully | Online |
| BIOS virtualization enabled | systeminfo confirms Hyper-V requirements available | Online |
| Default paths configured | Approved VM and VHDX paths saved in Hyper-V Settings | Online |
| Virtual switch created | Correct NIC binding and switch naming verified | 6 created |
| Event log healthy | No critical Hyper-V initialization errors present | Online |
| Test VM successful | Host can create and start a validation VM | Online |

# 8. Troubleshooting Guidance

### Hyper-V role will not install

Verify Windows edition support, BIOS virtualization, and that third-party hypervisors are not conflicting with the platform.

### No external connectivity through vSwitch

Confirm the correct physical NIC is selected, driver health is normal, and management OS sharing is configured appropriately.

### VM creation fails due to storage

Validate folder paths, available free space, NTFS permissions, and whether Controlled Folder Access or endpoint policy is blocking writes.

### Host shows Hyper-V service errors

Review Event Viewer under Hyper-V-VMMS and Hyper-V-Hypervisor, then validate required services are present and automatic startup is functioning.

# 9. Documentation and Reporting

Upon completion, record the following in the build log or asset inventory:

Hostname

Static IP address or management network assignment

Hyper-V storage paths

Virtual switch names and physical NIC mappings

Windows version and patch level

Technician name, date, and any deviations from standard

Any deviation from this SOP must be documented and approved by the instructor, system administrator, or other designated authority before the host is used for lab operations.

# 10. Revision History

| Version | Date | Author | Revision Notes |
| --- | --- | --- | --- |
| 1.0 | 2024-09-12 | Nicholas Jones | Initial mixed draft |
| 1.1 | 2025-10-06 | Caleb Roller / lab edits | Added content to working draft |
| 2.0 | 2025-10-06 | Joshua Sartain | Reframed as Hyper-V host installation and initial setup SOP; removed endpoint lifecycle scope drift |

# 11. Approval and Acknowledgment

| Role | Name | Signature / Date |
| --- | --- | --- |
| Prepared by | Joshua Sartain | 2025-10-06 |
| Reviewed by | Christian Word | 2025-10-06 |
| Approved by | Cherish Adams | 2025-10-06 |

# Appendix A - Training Record

Use this page to document personnel training on this SOP. Training must be repeated after substantive revisions.

| Name | Signature | Trainer Initials | Date |
| --- | --- | --- | --- |